INDEX

Embedded and IoT Technology

| Sr No | Practical | Date | Sign |
| --- | --- | --- | --- |
| 1 | Design and implement basics embedded circuits 1. Automatic Alarm system- Alarm should get trigger by senor 2. Timer based buzzer 3. Sensor based Counting device |  |  |
| 2 | Demonstrate communication between two embedded devices using UART port. |  |  |
| 3 | Build an IoT system to send ticket before entering the bus. |  |  |
| 5 | Develop a IoT application which will record the movement and orientation of your phone and give the data back to the PC |  |  |
| 6 | Develop an IoT application that will raise an alarm whenever with going to rain outside based on the weather prediction data. |  |  |
| 7 | Deploy an IoT application which will alert you by beeping or vibrating your phone whenever you get someone call your name. |  |  |
| 8 | Develop an IoT application for monitoring water levels in tanks and automatically start the motor to fill the tank if the level goes below critical level. |  |  |
| 9 | Develop an IoT module to which measure the intensity of light and send the same to your PC/phone. |  |  |
| 10 | Develop an IoT application for Motion detection. |  |  |

Practical 1

Aim:  Design and implement basics embedded circuits

1. Automatic Alarm system- Alarm should get trigger by senor

Keep Keil folder in C: & Proteus folder in D:

Keil:

Click on Project  New uVision Project

Type code & save it with .c extension

Go to Target  Source group1  right click  select add existing files

Select file & click Add

Right click on target  Option for target  Change frequency to 11.0592 MHz  Click on use on-chip ROM  Go to output label  Click on Create Hex file  Ok

Click on Project  Build Target

Proteus:

Click P  following window will open, add devices

- AT89C51

- Buzzer active DC Operated Buzzer Output Via Sound Card

- LCD: LM016L 16x2 Alphanumeric LCD

- Button (SPST Push button)

Go  to Schematic Capture  Drag and drop all devices  click on Terminals mode  add power & ground

Design the Circuit

Double click on Micro controller  Select HEX file form program file  Change Clock frequency to 11.0592MHz  Click Ok

Output:

Start the simulation

2. Timer based buzzer

Keil:

Proteus:

Create new project  add following devices

- AT89C51

- Buzzer active DC Operated Buzzer Output Via Sound Card

- Speaker- loudspeaker model(analog)- output via sound card

- Potentiometer- POT-HG

- Low-frequency low-power silicon NPN Transistor

- DC voltmeter from instruments

Design the Circuit

Double click on Micro controller  Select HEX file form program file  Change Clock frequency to 11.0592MHz  Click Ok

Output: 	Start the simulation

3. Sensor based Counting device

Keil:

#include<reg51.h>

sbit sen1=P1^0;        //entry

sbit sen2=P1^1; //exit

sbit rs=P1^2;

sbit rw=P1^3;

sbit en=P1^4;

void lcdcmd(unsigned char);

void lcddat(unsigned char);

void delay();

void lcddis(unsigned char *s,unsigned char r);

void lcdconv(unsigned char);

void main()

{

unsigned char x,y;

lcdcmd(0x38);

delay();

lcdcmd(0x01);

delay();

lcdcmd(0x10);

delay();

lcdcmd(0x0c);

delay();

lcddis("WELCOME",7);

lcdcmd(0xc0);

delay();

lcddis("VISITOR COUNTER",15);

delay();

lcdcmd(0x01);

delay();

while(1)

{

if(sen1==0)

{

lcdcmd(0x80);

delay();

lcddis("ENTRY:",6);

lcdcmd(0x87);

delay();

x=x+1;

lcdconv(x);

delay();

}

if(sen2==0)

{

lcdcmd(0xc0);

delay();

lcddis("EXIT:",5);

lcdcmd(0xc6);

delay();

y=y+1;

lcdconv(y);

delay();

}

}

}

void lcdcmd(unsigned char val)

{

P2=val;

rs=0;

rw=0;

en=1;

delay();

en=0;

}

void lcddat(unsigned char dat)

{

P2=dat;

rs=1;

rw=0;

en=1;

delay();

en=0;

}

void lcddis(unsigned char*s,unsigned char r)

{

unsigned char w;

for(w=0;w<r;w++)

{

lcddat(s[w]);

delay();

}

}

void lcdconv(unsigned char num)

{

unsigned char p,n;

p=num/10;

n=num%10;

p=num+0x30;

n=n+0x30;

lcddat(p);

lcddat(n);

}

void delay()

{

unsigned int k,l;

for(k=0;k<1000;k++);

for(l=0;l<100;l++);

}

Proteus:

Create new project  add following devices

- AT8C51

- LCD: LM016L 16x2 Alphanumeric LCD

- Button (SPST Push button)

Design the Circuit

Double click on Micro controller  Select HEX file form program file  Change Clock frequency to 11.0592MHz  Click Ok

Output: 	Start the simulation

Practical 2

Aim:  Demonstrate communication between two embedded devices using UART port.

Keil:

Proteus:

Create new project  add following devices

- AT8C51

- Virtual Terminal from instruments

Design the Circuit

Output: 	Start the simulation

Practical 3

Aim:  Build an IoT system to send ticket before entering the bus.

Arduino IDE:

Select Arduino Uno Board

Proteus:

Create new project  add following devices

- SIMULINO UNO controller

- MOTOR- PWMSERVO -Animated PMW Controlled Servo motor

- Virtual Terminal from instruments.

Design the circuit:

Output:	Run the simulation

Practical 5

Aim:  Develop a IoT application which will record the movement and orientation of your phone and give the data back to the PC.

Go to https://appinventor.mit.edu/  click on Get started  click on ‘online tool’  create new project

From palette  go to sensor  select orientation sensor  go to drawing and animation  select canvas  take a ball and put it in canvas

Set the properties

Click on Blocks  OrientationSensor1  click on when block  click on Ball1  select call Ball1.Move to block

Click on Ball1  select Ball1 X and Ball1 Y block

Final block diagram

Save your project  Click on Build  Android App (.apk)

Click on connect  AI companion

Open MIT App inventor on mobile  scan this QR code on mobile

Download prac5 apk and open it to see the output

Practical 6

Aim:  Develop an IoT application that will raise an alarm whenever with going to rain outside based on the weather prediction data.

Design the circuit in cisco packet tracer

Click on MCU-PT  Programming  click on New  Create a new file as rain_detector.js  Write following program

Click on Run  see the output

Here, we can see when humidity level is greater than 500 i.e., when it is raining outside the Alarm is turned ON.

Practical 7

Aim:  Deploy an IoT application which will alert you by beeping or vibrating your phone whenever you get someone call your name.

Go to MIT App Inventor  Projects  Start new project  give project name  click OK

Go to User Interface  select Button  then select Label

Go to Media  select Speech Recognizer  then select Sound

Draw the block diagram as follows

Click on Projects  save project

Click on Build  Android app (.apk)  then scan the code and get output  click on button  As you speak your name your phone gets vibrate

Practical 8

Aim:  Develop an IoT application for monitoring water levels in tanks and automatically start the motor to fill the tank if the level goes below critical level.

Proteus:

Create new project  add following devices

- 2N3904 – 4 placed, Silicon NPN Low Power High Frequency Bipolar Transistor

- LED GREEN – 4 placed, Animated LED model (Green)

- LOGICTOGGLE – 4 placed, Logic State Source (Momentary Action)

Design the circuit:

Right Click on Water Tank  go to child sheet  design the logic:

Output:	Start the simulation

Practical 9

Aim:  Develop an IoT module to which measure the intensity of light and send the same to your PC/phone.

MIT App inventor

Block diagram

Output:

Practical 10

Aim:  Develop an IoT application for Motion detection.

Open Cisco Packet Tracer  design the circuit  take components Motion sensor, MCU-PT and Light  D0 pin of MCU connect with D0 pin of Motion sensor, D1 pin of MCU connect with D0 pin of light using IOT custom cable.

Click on MCU-PT  click on Programming  New  give name as “motion_detection.visual”  click Create

Click on run button and run the program  Click Alt and move mouse on the motion sensor and view the output of Light.

Output: